This document is based on a free document supplied by The Legal Stop Limited through its website www.legalstop.co.uk

[PLEASE RETAIN THE CREDIT AND HYPERLINK IN ACCORDANCE WITH THE TERMS OF THE LICENCE UNDER WHICH THE TEMPLATE IS PROVIDED.  THE HYPERLINK SHOULD POINT TO: www.legalstop.co.uk YOU CAN HOWEVER DELETE THIS NOTE BETWEEN SQUARE BRACKETS]

WORKPLACE TRANSPORT SAFETY CHECKLIST

The following checklist is designed to guide employers when assessing workplace transport hazards. This is not an exhaustive list and may not be extensive for all workplace situations. Please place an ‘X’ in the relevant box and detail necessary actions at the end of the document.

| VEHICLES |  |
| --- | --- |
| MAINTENANCE Are vehicles maintained properly? i.e.: | X |
| There is a regular preventive maintenance programme for each vehicle that is carried out at set times or mileages. |  |
| Drivers carry out basic safety checks before using vehicles. |  |
| A thorough examination is conducted when vehicles lift people or materials. |  |
| VEHICLE SUITABILITY Are vehicles safe and suitable for the work they are being used for? i.e.: | X |
| Vehicles are provided with reflectors, horns, lights, reversing lights and other safety features, as necessary. |  |
| Vehicles have seats and restraints where necessary and are safe and comfortable for the persons using them. |  |
| Vehicles have suitable external mirrors and additional aids (for example, CCTV) where necessary to provide greater visibility when manoeuvring. |  |
| Drivers are protected against bad weather conditions, or against an unpleasant working environment. Please consider extremes of temperature, dirt, dust, fumes, and excessive noise and vibration. |  |
| Suitable vehicles and attachments have been chosen for the tasks that are actually carried out. |  |
| There are guards on dangerous parts of the vehicles, such as power take-offs, chain drives, exposed exhaust pipes etc. |  |
| Drivers are protected against being hit by falling objects. Is the protection in place and strong enough? |  |
| Vehicles have suitable and effective service and parking brakes. |  |
| Drivers are protected against injury in case the vehicle overturns. Is the protection in place and strong enough? |  |
| There is a safe way of getting into and out of the driving position, and to or from any other parts that need to be reached. |  |
| THE WORKPLACE |  |
| SAFETY FEATURES Are suitable safety features provided where appropriate? i.e.: | X |
| Features, such as fixed mirrors, have been put in place to provide greater vision at blind bends, road humps to reduce vehicle speeds, or barriers to keep vehicles and pedestrians apart. |  |
| Roadways marked where necessary (for example, to show the right of way at road junctions) |  |
| Visiting drivers and pedestrians report at the entrance for instructions before entering the site. |  |
| Road signs as set out in the Highway Code are used where necessary. |  |
| ROUTE LAYOUT | X |
| There are suitable parking areas for all parking needs. |  |
| A one-way system is used on vehicle routes within the workplace where this can be achieved. |  |
| Vehicles and pedestrians are kept safely apart. |  |
| Vehicle routes avoid sharp or blind bends. |  |
| There are suitable pedestrian crossing points on vehicle routes. |  |
| VEHICLE TRAFFIC ROUTES Routes need to be suitable for the type and quantity of vehicles that use them. i.e.: | X |
| Traffic routes are well maintained. |  |
| Traffic routes are free from obstructions and other hazards. |  |
| Vehicles are wide enough for the traffic routes. |  |
| Traffic routes have firm and even surfaces. |  |
| VEHICLE ACTIVITIES |  |
| LOADING/UNLOADING: Is loading and unloading carried out safely? i.e.: | X |
| Loading and unloading is carried out using safe systems of work on ground that is flat, firm, and free from potholes. |  |
| Loading and unloading is carried out so that, as far as possible, the load is spread evenly to prevent the vehicle or trailer from becoming unstable. |  |
| Checks are made to ensure that vehicles are not loaded with more material than they can carry. |  |
| Loading and unloading is carried out in an area away from passing traffic, pedestrians and others not involved in the work. Are you sure that people are not needlessly 'steadying' the load? |  |
| Checks are made to ensure that loads are secured and arranged so that they cannot move about. |  |
| The need for people to go on the load area of the vehicle has been removed where possible. |  |
| Vehicles are braked and stabilised as appropriate, to prevent unsafe movement during loading and unloading. |  |
| There is enough co-operation between the site occupier and duty holders who deliver or pick up goods. For example, are all the loads, the delivery vehicles, the vehicles that handle materials and the equipment compatible? |  |
| TIPPING: Is tipping carried out safely? i.e.: | X |
| Drivers are experienced enough to anticipate loads sticking. |  |
| Tipping happens on grounds that are level and stable, and at a place that is clear of overhead obstructions such as power lines, pipework etc. |  |
| Drivers always ensure that the body is completely empty, and drive no more than a few metres forward to make sure the load is clear. |  |
| Visiting drivers report to the site manager for any relevant instructions before tipping. |  |
| Suitably sized wheel stops are provided where vehicles need to reverse before tipping. |  |
| Where sites are not level and stable, the tipping faces are safe for vehicles involved in tipping. For example, are they compacted and with no significant side slopes? |  |
| Non-essential staff are kept well-clear of tipping areas. |  |
| REVERSING: Is reversing kept to a minimum and, where reversing is necessary, is it done safely and in safe areas? i.e.: | X |
| Non-essential staff are kept clear from areas where reversing is common. |  |
| Where there is an unavoidable need to use a banksman (a signaller) to direct reversing vehicles, they are adequately trained and visible, and otherwise able to work safely. |  |
| Vehicles have reversing alarms. |  |
| There is a one-way system on routes to reduce the need for reversing manoeuvres. |  |
| Additional visibility aids, e.g. CCTV, have been fitted to vehicles to eliminate or reduce blind spots during reversing. |  |
| You have identified and marked "reversing areas" to be clear to both drivers and pedestrians. |  |
| Side-mounted and rear-view mirrors are attached to the outside of the vehicles to provide the best all-round visibility. |  |
| SHEETING/UNSHEETING: Is sheeting and unsheeting carried out safely? i.e.: | X |
| There is room for using mechanical sheeting systems to avoid the need for manual sheeting. |  |
| Gloves, safety boots and, where necessary, eye and head protection are provided and used. |  |
| Sheeting and unsheeting is carried out in a safe place, away from passing traffic and pedestrians and sheltered from strong winds and bad weather. |  |
| Where manual sheeting is unavoidable, there is a system in place that avoids the need for a person to climb on the vehicle or load, for example by sheeting from the ground or by providing a platform from which loads can be sheeted. |  |
| Vehicles are parked on level ground, with their parking brakes on and the ignition key removed. |  |
| COUPLING/UNCOUPLING: Is coupling and uncoupling carried out safely? i.e.: | X |
| The platform area behind the tractive unit is kept clear, as clean as possible, and well-lit, to help prevent falls. |  |
| Drivers are experienced enough to know whether the fifth wheel is working properly. |  |
| Drivers wear high-visibility clothing when they work around other moving vehicles. |  |
| The semi-trailer parking brake is always applied before the hoses are disconnected, instead of using the emergency brakes to secure the trailer. |  |
| Gloves and safety boots are provided and used where necessary. |  |
| Landing legs are always fully extended, properly padded and locked in place as necessary. |  |
| Drivers know how to move any adjustable fairings to get behind their tractive unit easily, and are they doing so. |  |
| Drivers check that the ground is able to support the semi-trailer and landing legs before uncoupling. |  |
| PARKING: Do your drivers take care when parking their vehicles, including their own private cars, and that they park in safe places? i.e.: | X |
| Do drivers use the parking areas? |  |
| Drivers always put the brakes on for their vehicles and trailers and secure them before leaving them parked. |  |
| DRIVERS AND OTHER PEOPLE |  |
| DRIVERS/ EMPLOYEES: What do your drivers and other employees actually do when carrying out their work activities? | X |
| Are you sure that there is no risk of accidents caused by tiredness as a result of working long hours? |  |
| Are you sure that your drivers and other employees do not have to rush to complete their work on time? |  |
| Are you sure that your drivers drive with care? For example, do they use the correct routes, drive within the speed limit, and follow any other site rules? |  |
| Do managers and supervisors routinely challenge and investigate unsafe behaviour? ·Do managers and supervisors set a good example, i.e., by following instructions to separate vehicles and pedestrians, and by wearing high-visibility clothing where needed? |  |
| Are you sure that your employees are working safely? Consider how they go about loading and unloading, securing loads, carrying out maintenance etc. |  |
| Are you sure that your drivers and employees park safely and in safe places? |  |
| CONSULTATION: Check in consultation with your employees, that your level of management control and supervision is suitable. i.e.: | X |
| Do you take adequate steps to assess the behaviour of pedestrians and drivers of site and visiting vehicles, to investigate any underlying reasons for unsafe behaviour, and to correct this? |  |
| Are your supervisors, drivers, and other employees (including contractors and visiting drivers) aware of the site rules on using vehicles and traffic movement? Are they aware of their responsibilities to maintain a safe workplace and work safely? |  |
| Have you made certain people responsible for detecting, investigating, and correcting unsafe behaviour? |  |
| Is everyone at the workplace supervised and held accountable for their responsibilities, and is a clear system of penalties enforced when employees, contractors etc. fail to maintain standards? |  |
| TRAINING Does your selection and training procedures ensure that your drivers and other employees are able to work safely and responsibly? i.e.: | X |
| You provide training on how to do the job, and information about particular hazards, speed limits, the appropriate parking and loading areas, etc. |  |
| You have a planned programme of refresher training for drivers and other employees. |  |
| You check the previous experience of your drivers and test them to make sure that they are competent. |  |

FURTHER ACTION/ ADDITIONAL INFORMATION (Please specify)